# Lesson 5 Summary: Hard Drives, SSDs & Storage Troubleshooting

## What You Will Learn

By the end of this lesson, you should be able to:

- Explain how hard drives and SSDs work and how computers communicate with them

- Install and configure storage devices

- Use an organized troubleshooting approach for hardware issues

- Troubleshoot hard drive failures

- Support optical drives, SSDs, and flash memory devices

CompTIA A+ Core 1 Objectives Covered:
1.1 (Mobile device hardware), 3.1 (Display components), 3.3 (RAM characteristics), 5.3 (Troubleshooting video/display issues)

# 1. Overview of Storage Device Support

Storage devices are essential for holding OS files, applications, media, and user data. Supporting these devices includes upgrading, installing, troubleshooting, securing, and optimizing performance.

# 2. Hard Disk Drives (HDDs)

HDD = magnetic, mechanical storage with spinning platters and read/write heads.

### HDD Characteristics

- Form Factors:

  - 3.5-inch — desktops

  - 2.5-inch — laptops

- Interface Types:

  - SATA (most common today)

  - Older: PATA/EIDE

  - Enterprise: SAS (high speed, high durability)

- Performance Factors:

  - Rotational speed (rpm)

  - Cache size

  - SATA generation (e.g., SATA III = 6 Gb/s)

# 3. Solid-State Drives (SSDs)

SSDs use NAND flash memory and have no moving parts.

### Advantages

- Much faster than HDDs

- More durable (shock-resistant)

- Lower power usage

- Silent operation

### How SSDs Work

Data stored in memory cells using electrical charges.

### SSDs Are Faster Because:

- No mechanical movement

- Can access data instantly

- NVMe + PCIe = massively higher speeds than SATA

# 4. SSD Features

### Form Factors

- 2.5-inch SATA SSD – easy HDD replacement

- M.2 SSD – very small, mounts on motherboard directly

- PCIe Expansion Card SSDs – very high performance

### Connection Types

- SATA – up to 6 Gb/s

- NVMe – designed for SSDs; uses PCIe lanes

- PCIe – direct high-speed connection to motherboard

NVMe + PCIe = up to 4× faster than SATA SSDs

# 5. Installing a Hard Drive

Steps:

- Power off the system and unplug it

- Ground yourself (anti-static)

- Mount the drive in a drive bay

- Connect SATA data cable to motherboard

- Connect SATA power cable from PSU

- Boot into BIOS/UEFI → confirm drive is detected

- Close the case and manage cables

- Initialize, partition, and format the drive in OS

# 6. Removable Storage & Security

### Flash Drives (USB Drives)

- Portable

- Use flash memory

- Ideal for file transfer and backups

### Memory Cards

- SD, microSD, CF

- Used in cameras, phones, gaming devices

### Optical Drives

- CD, DVD, Blu-ray

- Used for media playback, software installation, backups

- Less common today due to cloud and flash storage

### Cloud Storage

- Online services (OneDrive, Google Drive, Dropbox)

- Useful for syncing, sharing, and backups

### Security Reminder

Removable storage can be lost or stolen — always use:

- Encryption

- Password protection

- Backups