Lesson 3

After completing this lesson, you will be able to:

- Compare characteristics and features of processors used for personal computers.

- Select, install, and upgrade a processor.

- Compare the different kinds of physical memory and how they work.

- Upgrade memory.

A central processing unit (CPU) is the primary component of a computer that carries out the instructions of a computer program by performing basic arithmetic, logic, control, and input/output (I/O) operations.

CPU Architecture: Defines how a processor works, including its structure and instructions. Common types are x86/x64 (PCs, laptops, servers) and ARM (mobile devices, tablets, Apple products).

x86/x64 (Intel & AMD):

- x86 = 32-bit; x64/x86-64 = 64-bit, supports more memory.

- Features: rectangular package, pins or pads, heatspreader, specific motherboard socket, integrated cores and cache.

- AMD offers compatible processors (Ryzen, Threadripper) often at lower cost.

ARM:

- Energy-efficient, simple design (RISC), used in smartphones, tablets, some laptops.

- Often integrated into a single SoC with CPU, GPU, memory controllers, and other components.

Apple Silicon:

- Custom ARM-based SoCs for Apple devices.

- Integrates CPU, GPU, Neural Engine, memory, and storage into one chip.

- Not upgradeable or replaceable individually.

Key point: CPU choice depends on architecture, cores, threading, and compatibility with the motherboard.

## Cooling Options & CPU Cooler Installation (Summary)

Cooling Options:

- Air cooling

- AIO liquid cooling

- Passive cooling

- Case fans

How to Install a Fan-Based CPU Cooler:

- Gather cooler, thermal paste, tools.

- Install mounting bracket.

- Install CPU.

- Apply a small pea-sized amount of thermal paste (if needed).

- Place cooler flat on CPU and align screws/clips.

- Tighten screws gently in a diagonal pattern.

- Plug cooler fan into the CPU fan header.

- Organize cables for airflow.

- Power on and check CPU temperatures.

## DIMMs & RAM Installation (Summary)

What is a DIMM?

- A memory module used in desktops/servers.

- “Dual” = pins on both sides.

- All DIMMs use a 64-bit data path.

- DDR memory transfers data on both clock edges.

Common Types:

- SO-DIMM (laptops)

- DDR3

- DDR4

- DDR5

- ECC RAM (error-correcting memory)

How to Install RAM:

- Confirm RAM type is compatible with the motherboard.

- Power off and unplug the PC.

- Open the case and locate RAM slots.

- Open retention clips.

- Align RAM with the slot notch.

- Press firmly until clips lock.

- Install pairs in matching slots (if needed).

- Check that all modules are seated properly.

- Boot PC and verify RAM in BIOS/OS.

- Run a memory test for errors.

Lesson 3 covered the key topics of supporting processors and upgrading memory. We explained the role of the CPU as the "brain" of the computer and discussed its components, such as the arithmetic logic unit (ALU), control unit, and registers. Next, we outlined various CPU architectures, including x86, x86-64, and ARM, highlighting their characteristics, including CPU features like cores, threads, clock speeds, cache, and virtualization. Finally, we provided insights into processor form factors, CPU installation and cooling methods, and different RAM technologies, and offered step-by-step instructions for RAM installation and upgrades.

This lesson covered the different types and features of CPU processors and how to install processors. Learners evaluated the different cooling options and how to install these, as wel las the different types of RAM and how to install and upgrade these.